import bpy

import os

import math

import xml.etree.ElementTree as ET

# --- CONFIGURATION ---

kml_path = r'C:\Users\karol\Desktop\3DCityDB_Exporter_exports\Collada\tcim_malmo_xl_TIN\Flood_BD\Tiles\0\0\tcim_malmo_xl_Flood_BD_Tile_0_0_collada.kml'

dae_folder = r'C:\Users\karol\Desktop\3DCityDB_Exporter_exports\Collada\tcim_malmo_xl_TIN\Flood_BD_all_in_one\buildings' # Folder where building DAEs are

# ---------------------

def get_meters_per_degree(lat):

lat_rad = math.radians(lat)

# Metric approximation for Sweden's latitude

m_per_lat = 111132.92 - 559.82 * math.cos(2 * lat_rad)

m_per_lon = 111412.84 * math.cos(lat_rad)

return m_per_lat, m_per_lon

# Parse KML

tree = ET.parse(kml_path)

root = tree.getroot()

# Find all Placemarks

placemarks = root.findall('.//{*}Placemark')

if not placemarks:

print("Error: No Placemarks found.")

else:

# 1. Get Terrain Anchor (Last Placemark)

terrain_pm = placemarks[-1]

t_lon = float(terrain_pm.find('.//{*}longitude').text)

t_lat = float(terrain_pm.find('.//{*}latitude').text)

t_alt = float(terrain_pm.find('.//{*}altitude').text)

m_per_lat, m_per_lon = get_meters_per_degree(t_lat)

print(f"Origin set at Terrain: {t_lat}, {t_lon}")

# 2. Process Buildings (all except the last placemark)

for pm in placemarks[:-1]:

href_tag = pm.find('.//{*}href')

if href_tag is not None:

# os.path.basename handles cases like '27030/filename.dae'

filename = os.path.basename(href_tag.text)

dae_path = os.path.join(dae_folder, filename)

if os.path.exists(dae_path):

# Extracting nested tags

lon = float(pm.find('.//{*}longitude').text)

lat = float(pm.find('.//{*}latitude').text)

alt = float(pm.find('.//{*}altitude').text)

head = float(pm.find('.//{*}heading').text)

# Calculate relative distance from terrain anchor

x = (lon - t_lon) * m_per_lon

y = (lat - t_lat) * m_per_lat

# Import the DAE

bpy.ops.wm.collada_import(filepath=dae_path)

# Position and rotate the imported building

for obj in bpy.context.selected_objects:

obj.location = (x, y, alt)

# KML heading is degrees clockwise; Blender Z is radians counter-clockwise

obj.rotation_euler = (0, 0, math.radians(-head))

else:

print(f"File not found: {filename}")

print("All buildings aligned.")